(True or False) For each of the following statements, if the statement is true, put the letter T (for true) in the blank before the statement.  If the statement is false, put the letter F (for false) in the blank.

1. _T__ Any digital circuit can be described by a truth table

2. __T _ Any truth table can be implemented with a digital circuit (built so that the output of the digital circuit is the same as the truth table.

3. _F__ The SOP (sum of products) implementation of a circuit usually gives the smallest circuit.

4. _T__ We can use Boolean algebra to reduce the complexity of circuits.

5. _F__ The Duality Theorem for Boolean algebra says that for every true Boolean algebra theorem, if you simply replace all multiplication (*) with addition (+) and replace all addition with multiplication, another true theorem will be formed.

6. _T__ Minterms describe the conditions of the variables for each line of the truth table.

7. _F__ The SOP (sum of products) implementation of a circuit is the Boolean sum of each of the minterms that makes the function false.

For each of the following two-input logic gates, indicate which Boolean operation is performed by that gate.  Indicate which is the equivalent by choosing a letter from the following table (one letter will be used twice):

| Letter | Equivalent Boolean Operation |
| --- | --- |
| A | Boolean negation |
| B | The equivalent Boolean operation for this gate is more complex than the other operations listed |
| C | Boolean multiplication |
| D | Negation of the Boolean product (multiplication) |
| E | Boolean addition |
| F | Boolean product (multiplication) of the negations |

8. _C__ AND gate.

9. _E__ OR gate.

10. __A_ NOT gate

11. __B_ XOR gate

12. _D__ NAND gate

13. __F_ NOR gate

14. _B__ XNOR gate