1. (True or False) For each of the phrases below, decide if the phrase completes the sentence to make a true statement or a false statement.  If it forms a true statement, put the letter T (for true) in the blank before the phrase.  If it forms a false statement, put the letter F (for false) in the blank.

In digital electronics,

- _____ transistors are normally operated in cutoff or in saturation.

- _____ we cannot use the same transistors as in linear electronics.

- _____ BJTs are sometimes used, and field effect transistors (FETs) are sometimes used.

- _____ the outputs are normally either in a “high” state or a “low” state.

(True or False) For each of the following statements, if the statement is true, put the letter T (for true) in the blank before the statement.  If the statement is false, put the letter F (for false) in the blank.

2. ____ A PMOS transistor acts like an open circuit when the gate voltage is high (near the supply voltage) and acts like a closed circuit when the gate voltage is low (near zero volts).

3. ____ An NMOS transistor acts like an open circuit when the gate voltage is high (near the supply voltage) and acts like a closed circuit when the gate voltage is low (near zero volts).

4. ____ If logic chips of different types (such as TTL and CMOS) are not in a circuit together, we do not need to know the inner workings of the different types of chips.

5. ____ All logic gates have two or more inputs.

6. ____ Boolean algebra allows us to ignore the inner workings of the logic gates.

7. ____ For digital electronics, a “high” output means that the output voltage is one volt.

Each of the symbols below represents a different type of logic gate.  Put the name of the gate next to the symbol.

x1

x2

output

______

x1

output

x2

______

x1

x2

output

______

x1

x2

output

______

x1

output

______

x1

x2

output

______

x1

x2

output

______

Each of the symbols below represents a different type of logic gate.  Fill out the truth table for each gate.

x1

x2

output

| x1 | x2 | output |
| --- | --- | --- |

x1

output

x2

| x1 | x2 | output |
| --- | --- | --- |

x1

x2

output

| x1 | x2 | output |
| --- | --- | --- |

x1

x2

output

| x1 | x2 | output |
| --- | --- | --- |

x1

output

| x1 | output |
| --- | --- |

| x1 | x2 | x1 x2 output output |
| --- | --- | --- |

x1

x2

output

| x1 | x2 | output |
| --- | --- | --- |

(True or False) For each of the following statements, if the statement is true, put the letter T (for true) in the blank before the statement.  If the statement is false, put the letter F (for false) in the blank.

1. ____ Any digital circuit can be described by a truth table

2. ____ Any truth table can be implemented with a digital circuit (built so that the output of the digital circuit is the same as the truth table.

3. ____ The SOP (sum of products) implementation of a circuit usually gives the smallest circuit.

4. ____ We can use Boolean algebra to reduce the complexity of circuits.

5. ____ The Duality Theorem for Boolean algebra says that for every true Boolean algebra theorem, if you simply replace all multiplication (*) with addition (+) and replace all addition with multiplication, another true theorem will be formed.

6. ____ Minterms describe the conditions of the variables for each line of the truth table.

7. ____ The SOP (sum of products) implementation of a circuit is the Boolean sum of each of the minterms that makes the function false.

For each of the following two-input logic gates, indicate which Boolean operation is performed by that gate.  Indicate which is the equivalent by choosing a letter from the following table (one letter will be used twice):

| Letter | Equivalent Boolean Operation |
| --- | --- |
| A | Boolean negation |
| B | The equivalent Boolean operation for this gate is more complex than the other operations listed |
| C | Boolean multiplication |
| D | Negation of the Boolean product (multiplication) |
| E | Boolean addition |
| F | Boolean product (multiplication) of the negations |

8. ____ AND gate.

9. ____ OR gate.

10. ____ NOT gate

11. ____ XOR gate

12. ____ NAND gate

13. ____ NOR gate

14. ____ XNOR gate