- (Multiple choice) In a diode, the current that is due to the drift of electrons in the built-in electric field is

  - from the “p” area to the “n” area

  - from the “n” area to the “p” area

- (Multiple choice) In a diode, the current that is due to the drift of holes in the built-in electric field is

  - from the “p” area to the “n” area

  - from the “n” area to the “p” area

- (Multiple choice) In a diode, the current that is due to the diffusion of electrons is

  - from the “p” area to the “n” area

  - from the “n” area to the “p” area

- (Multiple choice) In a diode, the current that is due to the diffusion of holes is

  - from the “p” area to the “n” area

  - from the “n” area to the “p” area

- (Multiple choice) A forward biased diode conducts current because

  - the drift current is increased

  - the drift current is decreased

  - the diffusion current is increased

  - the diffusion current is decreased

- (Multiple choice -- select all choices which create a true statement:) 
A reverse biased diode blocks the flow of current because

  - the increased electric field causes the drift current to decrease

  - the depletion of the minority carriers causes the drift current to decrease

  - the drift current is decreased, because the applied voltage causes a greater energy difference between the “p” and the “n” sides of the junction.

  - the diffusion current is decreased, because the applied voltage causes a greater energy difference between the “p” and the “n” sides of the junction.

  - the increased diffusion current depletes the minority carriers.

  - the depletion of the minority carriers causes the diffusion current to decrease.