- (Multiple Choice – circle all that are correct) What is the difference between Rin and Rin,B?

  - Rin is the input resistance of the amplifier, including the DC bias circuitry, and Rin,B is the input resistance at the base of the transistor (ignoring the bias circuitry).

  - Rin,B is the input resistance of the amplifier, including the DC bias circuitry and Rin is the input resistance at the base of the transistor (ignoring the bias circuitry).

  - Rin is the input resistance of the amplifier at the emitter and Rin,B is the input resistance at the base of the transistor.

- (Multiple Choice) For the common emitter amplifier with no emitter resistor, the output resistance is

  - Rc || rπ

  - Rc || ro

  - ro || rπ

  - ro || Rin