- (Matching) Match the two-transistor circuit name with the description.

Cascade circuit ______

Cascode circuit ______

Darlington pair ______

  - Two BJTs with the collectors joined together and the emitter current from the first transistor fed directly into the base of the second transistor

  - A common emitter transistor followed by another common emitter transistor.  The output of the first stage serves as the input to the second stage.

  - A common emitter transistor followed by a common base transistor.